TOBB University of Economics and Technology

Department of Electrical and Electronics Engineering

ELE 224L – Electronic Circuits Laboratory

Experiment 4 – Preliminary Work: BJT Circuits

| Name Surname |  |
| --- | --- |
| Student ID |  |
| Date |  |

1  INTRODUCTION

1.1  Objective of the Experiment

The objective of this experiment is to investigate the current–voltage characteristics of the bipolar junction transistor (BJT) and to learn the DC biasing methods. The BC238A transistor is used to find the operating point of two biasing circuits, to see how the collector current depends on VCE and on the base drive, and to compare two biasing schemes in terms of their sensitivity to the current gain β. The ability to diagnose wrong measurements of VCE is also practised.

1.2  Benefits of the Experiment

The experiment shows how a BJT is analysed with the assumed-region method (assume active region, calculate, verify the assumption, switch to saturation if the assumption fails), why a bias circuit with an emitter resistor is preferred, and how measured voltages reveal wiring faults. It also trains the correct reading of a datasheet and the correct placement of a TO-92 transistor on a breadboard. These skills are directly used in the BJT amplifier experiment.

2  CALCULATIONS AND SIMULATIONS

2.1  BJT Properties

The parameters of the BC238A were taken from the datasheet of the BC237/238/239 family (Fairchild Semiconductor), and they are listed in Table 2.1.1. The current gain of the class A transistors is specified as hFE=120–220 at VCE=5 V and IC=2 mA; the average value of 170 is used as β. The base–emitter turn-on voltage is typically 0.62 V (0.55–0.70 V) at the same bias. The collector–emitter saturation voltage is 0.2 V at most (IC=10 mA, IB=0.5 mA; typical 0.07 V), and 0.2 V is used as VCEsat in the calculations.

Table 2.1.1  BC238A parameters and TO-92 pin order from the datasheet

| Parameter | Value | Condition |
| --- | --- | --- |
| β (class A, average) | 170 (range 120–220) | VCE = 5 V, IC = 2 mA |
| VBEon (typical) | 0.62 V (min 0.55 V, max 0.70 V) | VCE = 5 V, IC = 2 mA |
| VCEsat (maximum) | 0.2 V (typical 0.07 V) | IC = 10 mA, IB = 0.5 mA |
| Pin 1 / Pin 2 / Pin 3 | Collector / Base / Emitter | TO-92, flat face towards the viewer, leads down |

The pin order is C–B–E for the Fairchild datasheet. Some other manufacturers use E–B–C for the same package, so the pin order of the physical part should be checked before the circuit is powered.

2.2  BJT Circuit 1 on the Breadboard

BJT circuit 1 (Figure 2.2.1) was drawn on the breadboard as shown in Figure 2.2.2. The transistor is placed with its three legs on three adjacent columns (collector on column 10, base on column 11, emitter on column 12). RC is connected between the collector column and a column that is wired to the +VCC rail, RE is connected between the emitter column and a column that is wired to the ground rail, and RB is connected between the base column and the VBB supply lead. The connections of the drawing were verified automatically against the circuit diagram.

Figure 2.2.1  BJT circuit 1 of the experiment (RB = 150 kΩ, RC = 2.2 kΩ, RE = 1 kΩ).

Figure 2.2.2  BJT circuit 1 drawn on the breadboard (VBB, VCC and GND points are indicated).

2.3  Q-Point Calculations of BJT Circuit 1

The operating point of circuit 1 was calculated for VBB=1, 2 and 5 V and for the VCC values of the experiment sheet with the model β=170, VBEon=0.62 V, VCEsat=0.2 V and no Early effect. The base-loop equation (2.1) gives the base current when the transistor is in the active region, and the remaining quantities follow from (2.2) and (2.3).

|  | VBB=IBRB+VBEon+IERE, IE=β+1IB ⇒ IB=VBB−VBEonRB+β+1RE | (2.1) |
| --- | --- | --- |

|  | IC=βIB, IE=β+1IB | (2.2) |
| --- | --- | --- |

|  | VCE=VCC−ICRC−IERE | (2.3) |
| --- | --- | --- |

The active-region assumption is verified by VCE≥VCEsat. If it fails, the transistor is in saturation: VCE=VCEsat, and IB and IC are found from the two loop equations given in (2.4); IC=βIB no longer holds. If the supply is so small that even this solution gives IC≤0, the collector current is practically zero and VCE=VCC.

|  | RB+REIB+REIC=VBB−VBEonREIB+RC+REIC=VCC−VCEsat | (2.4) |
| --- | --- | --- |

Sample calculation 1 (active region). For VBB=2 V and VCC=10 V: IB=2−0.62/150 kΩ+171⋅1 kΩ=4.299 µA, IC=170⋅4.299 μA=0.7308 mA, IE=0.7351 mA, and VCE=10−0.7308⋅2.2−0.7351⋅1=7.657 V > 0.2 V, so the assumption is valid. Sample calculation 2 (saturation). For VBB=5 V and VCC=4 V: the active-region result would be VCE=4−7.44<0.2 V, so the transistor is in saturation, and (2.4) gives IC=1.181 mA, VCE=0.2 V. The other cells are calculated in the same way.

Table 2.3.1  Table 1 values for VBB = 1 V (calculation and SPICE)

| VCC (V) | 0 | 0.2 | 0.5 | 1 | 2 | 4 | 6 | 8 | 10 | 15 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| VCE calc. (V) | 0 | 0.2 | 0.2 | 0.355 | 1.35 | 3.35 | 5.35 | 7.35 | 9.35 | 14.4 | 19.4 |
| IC calc. (mA) | 0 | 0 | 0.093 | 0.201 | 0.201 | 0.201 | 0.201 | 0.201 | 0.201 | 0.201 | 0.201 |
| VCE SPICE (V) | 0.0052 | 0.073 | 0.114 | 0.262 | 1.26 | 3.26 | 5.26 | 7.26 | 9.26 | 14.3 | 19.3 |
| IC SPICE (mA) | −0.0028 | 0.039 | 0.12 | 0.23 | 0.231 | 0.231 | 0.231 | 0.231 | 0.231 | 0.231 | 0.231 |

Table 2.3.2  Table 1 values for VBB = 2 V (calculation and SPICE)

| VCC (V) | 0 | 0.2 | 0.5 | 1 | 2 | 4 | 6 | 8 | 10 | 15 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| VCE calc. (V) | 0 | 0.2 | 0.2 | 0.2 | 0.2 | 1.66 | 3.66 | 5.66 | 7.66 | 12.7 | 17.7 |
| IC calc. (mA) | 0 | 0 | 0.091 | 0.248 | 0.561 | 0.731 | 0.731 | 0.731 | 0.731 | 0.731 | 0.731 |
| VCE SPICE (V) | 0.0088 | 0.05 | 0.076 | 0.101 | 0.146 | 1.61 | 3.61 | 5.61 | 7.61 | 12.6 | 17.6 |
| IC SPICE (mA) | −0.0059 | 0.044 | 0.13 | 0.279 | 0.578 | 0.744 | 0.744 | 0.744 | 0.744 | 0.744 | 0.744 |

Table 2.3.3  Table 1 values for VBB = 5 V (calculation and SPICE)

| VCC (V) | 0 | 0.2 | 0.5 | 1 | 2 | 4 | 6 | 8 | 10 | 15 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| VCE calc. (V) | 0 | 0.2 | 0.2 | 0.2 | 0.2 | 0.2 | 0.2 | 0.564 | 2.56 | 7.56 | 12.6 |
| IC calc. (mA) | 0 | 0 | 0.085 | 0.241 | 0.555 | 1.18 | 1.81 | 2.32 | 2.32 | 2.32 | 2.32 |
| VCE SPICE (V) | 0.012 | 0.033 | 0.05 | 0.067 | 0.088 | 0.117 | 0.149 | 0.571 | 2.57 | 7.57 | 12.6 |
| IC SPICE (mA) | −0.013 | 0.043 | 0.132 | 0.283 | 0.59 | 1.21 | 1.82 | 2.32 | 2.32 | 2.32 | 2.32 |

The results are plotted in Figure 2.3.1. In the active region (large VCC), IC is constant for a given VBB (0.201, 0.731 and 2.32 mA for VBB = 1, 2, 5 V) and VCE increases linearly with VCC, since the model has no Early effect; the three horizontal lines are the output characteristics of the transistor for three different base currents (IB=1.18, 4.30 and 13.6 µA). When VCC is decreased, the transistor enters the saturation region (the cells VCC≤6 V for VBB=5 V, ≤ 2 V for VBB=2 V and ≤ 0.5 V for VBB=1 V), where VCE is clamped at about 0.2 V and IC falls linearly with VCC; this is the steep part of the characteristic. For VCC=0, all currents are zero.

Figure 2.3.1  IC–VCE points of Table 1 (calculation: circles; SPICE: crosses); right: zoom on the saturation region.

The SPICE results (BC238A model with IS fitted to VBE=0.62 V at 2 mA, β=170, no Early effect; the model line is in Appendix A) are also given in the tables. In the active region, the largest difference in VCE is 0.0948 V (VBB = 1 V), 0.0432 V (2 V) and 0.00739 V (5 V), and the mean difference in IC is +14.6%, +1.8% and -0.1%, respectively. The difference is largest for VBB=1 V because the real VBE depends logarithmically on the current: at IC≈0.2 mA the real VBE is about 0.56 V, not 0.62 V, so the base current (which is proportional to VBB−VBE) is 15% larger than the calculated value. For VBB=5 V, the term VBB−VBE is large and the effect of the error in VBE disappears. In the saturation region, SPICE gives VCE between 0.03 and 0.15 V (it decreases with the base drive), whereas the model uses a constant 0.2 V; hence the saturation cells have a difference of up to 0.15 V in VCE and a few percent in IC.

2.4  Biasing Circuit 1 (No Emitter Resistor)

The circuit in Figure 2.4.1 has no emitter resistor. For VBB=5 V and VCC=12 V, the base current is IBQ=VBB−VBE/RB=5−0.62/150 kΩ=29.20 µA, the collector current is ICQ=βIBQ=4.964 mA, and the collector–emitter voltage is VCEQ=VCC−ICQRC=12−4.964×2.2=1.079 V, with VBEQ=0.62 V. Since VCEQ>0.2 V (and VCEQ>VBE, i.e., the collector junction is reverse biased), the active-region assumption is valid. The SPICE simulation gives VBEQ=0.6442 V, IBQ=29.04 µA, ICQ=4.937 mA and VCEQ=1.140 V; the differences from the calculation result from the dependence of VBE on the current (VBE=0.644 V at 5 mA).

Figure 2.4.1  BJT biasing circuit 1 (no emitter resistor).

The operating point of this circuit is very sensitive to β, because IC=βIB and IB is fixed by VBB and RB. With the datasheet range of β for the class A transistors, IC changes from 3.50 mA (β=120) to 5.36 (saturation limit) mA (β=220) in the active-region equations; for β=220, the active-region result would give VCE<0, so the transistor actually saturates with VCE=0.2 V and IC=5.36 mA, and the amplifier would be unusable. This is shown in the left graph of Figure 2.5.2.

2.5  Biasing Circuit 2 (With Emitter Resistor)

The circuit in Figure 2.5.1 has RB=22 kΩ, RC=1.2 kΩ and RE=1 kΩ. The calculation with (2.1)–(2.3) for VBB=5 V and VCC=12 V gives IBQ=4.38/22 kΩ+171 kΩ=22.69 µA, ICQ=3.858 mA, IEQ=3.881 mA, VBEQ=0.62 V, and VCEQ=12−3.858×1.2−3.881×1=3.490 V. The transistor is in the active region. The SPICE simulation gives VBEQ=0.6376 V, IBQ=22.60 µA, ICQ=3.843 mA and VCEQ=3.524 V.

Figure 2.5.1  BJT biasing circuit 2 (with emitter resistor).

Table 2.5.1  Sensitivity of the two bias circuits to β (VBB = 5 V, VCC = 12 V)

| β | Circuit 1: IC (mA) | Circuit 1: VCE (V) | Circuit 2: IC (mA) | Circuit 2: VCE (V) |
| --- | --- | --- | --- | --- |
| 120 | 3.50 | 4.29 | 3.68 | 3.88 |
| 170 | 4.96 | 1.08 | 3.86 | 3.49 |
| 220 | 5.36 | 0.200 (saturated) | 3.97 | 3.26 |

Figure 2.5.2  DC load lines and operating points of the two bias circuits for β = 120, 170 and 220 (calculation).

In circuit 2, the emitter resistor provides negative feedback: if IC tries to increase, the voltage drop on RE increases, the base–emitter voltage decreases, and IB decreases. As Table 2.5.1 shows, a change of β from 120 to 220 changes IC only from 3.68 to 3.97 mA (8%), whereas IC of circuit 1 changes by 37% and finally saturates. The operating point is stable because β+1RE=171 kΩ is large compared with RB, so that IC≈VBB−VBE/RE is almost independent of β.

2.6  Possible Reasons for Wrong VCEQ Readings

The reasons are listed item by item for biasing circuit 2 of Figure 2.5.1 (the correct value is VCEQ≈3.5 V). The values in Table 2.6.1 were obtained from SPICE simulations of each fault.

Table 2.6.1  SPICE results of the fault conditions in BJT biasing circuit 2 (VBB = 5 V, VCC = 12 V)

| Condition | VCE (V) | VBE (V) | IC (mA) |
| --- | --- | --- | --- |
| Normal operation (reference) | 3.52 | 0.638 | 3.84 |
| VBB = 0 V (base supply missing) | 12.0 | 0 | 0 |
| RB open (or base lead not inserted) | 12.0 | ≈ 0 (floating) | 0 |
| Collector and emitter pins exchanged | 11.4 | 0.564 | 0.185 |
| RE open (emitter floating) | 7.11 | 0.107 | 0 |
| RC open (collector floating) | 0.0179 | 0.583 | 0 |
| VCC = 0 V (collector supply missing) | 0.0107 | 0.576 | −0.0941 |
| RE shorted by a wire | 0.121 | 0.668 | 9.90 |
| RC = 12 kΩ instead of 1.2 kΩ | 0.0556 | 0.611 | 0.907 |
| Collector–emitter short circuit | ≈ 0 | −0.455 | 5.45 |

If VCEQ is measured as 12 V, no current flows through RC and RE (IC≈0), i.e., the transistor is in cut-off or the collector current path is broken. The possible reasons and the corresponding actions are:

• No base current: VBB is not applied or is 0 V (supply channel off, current limit at zero, ground wire not common). Action: measure VB with respect to ground; if it is 0 V, check the VBB channel and its wiring.

• Open RB or an unconnected base: a loose resistor leg, a hole in the wrong column, or a broken wire. Action: measure the voltage across RB (it should be about 0.5 V, i.e., IBRB=22.7 μA×22 kΩ); if VB=VBB and VBE≈0, the base is connected but the emitter path is open; if the voltage at the base side is 0 V, the base is not connected.

• Transistor inserted incorrectly or damaged (open base–emitter junction): a wrong pin order places the base on a wrong column. Action: check the pin order of the part (C–B–E), measure VBE (it should be 0.6–0.7 V when the transistor conducts), and replace the transistor if the junction does not conduct.

• Exchanged collector and emitter pins: the transistor works in the inverse mode with a very small current gain, and VCE is close to 12 V (11.4 V in the simulation, with VBE≈0.56 V and a small IC). Action: exchange the two pins.

• A measurement error: the voltmeter is connected between VCC and a point instead of between collector and emitter, or RE is open (then VCE is not 12 V but 7.1 V in the simulation, and VBE≈0.1 V, which distinguishes this case).

If VCEQ is measured as 0 V, the transistor is in deep saturation or there is a short circuit between the collector and the emitter. The possible reasons and the corresponding actions are:

• Excessive base drive or too large a collector resistor: RE shorted by a wire (0.121 V in the simulation, IC=9.9 mA), or a wrong RC value such as 12 kΩ (0.0556 V). Action: measure each resistor with an ohmmeter and compare with the colour code; remove the unintended wire.

• A short circuit between collector and emitter (a wire or a solder bridge on the breadboard, or a damaged transistor). In this case, VBE is typically negative or abnormal (−0.45 V in the simulation) and IC is about VCC/RC+RE=5.5 mA. Action: remove the short, or replace the transistor if it is damaged.

• VCC is not applied (0 V): the supply channel is off or its wire is disconnected (0.0107 V in the simulation, with a small reverse current in RC). Action: measure VCC at the top of RC.

• RC is open or not connected to the collector: the collector floats and the forward-biased collector junction pulls it to the emitter voltage (0.0179 V in the simulation, IC=0, VBE≈0.58 V). Action: check the continuity of RC and the column of the collector.

• A measurement error, such as both probes on the same node or the ground lead of the voltmeter on a floating ground. Action: repeat the measurement with the probes directly on the collector and emitter pins.

In every case, the systematic approach is to measure VB, VE and VC with respect to ground and to compare them with the expected values (VB≈4.50 V, VE≈3.88 V, VC≈7.37 V) in order to find the first node where the measurement deviates.

3  CONCLUSIONS

The calculated and the simulated results of Table 1 agree well in the active region when the base drive is large (VBB=5 V: the difference in IC is -0.1% and in VCE 0.00739 V), and the difference grows when the base drive is small (VBB=1 V: +14.6% in IC), because the constant VBEon=0.62 V of the model is valid only near 2 mA. In the saturation region, the constant VCEsat=0.2 V of the model is larger than the SPICE values (0.03–0.15 V), which is a limitation of the simple model rather than a calculation error. In the bias circuits, the circuit without RE has an operating point that depends strongly on β (and even saturates for β≥184, which is inside the datasheet range of 120–220), whereas the circuit with RE changes its collector current by only about 8% over the same range, which is why emitter feedback is used in practical amplifiers. The fault analysis showed that VCE=12 V and VCE=0 V can each arise from several different faults, and that the additional measurements of VBE and the node voltages are necessary to separate them.

4  REFERENCES

1. Fairchild Semiconductor. (2002). BC237/238/239 NPN epitaxial silicon transistor [Data sheet, Rev. A2]. Retrieved October 11, 2026, from https://wmsc.lcsc.com/wmsc/upload/file/pdf/v2/lcsc/2309151357_onsemi-BC237_C3201628.pdf

2. Sedra, A. S., & Smith, K. C. (2014). Microelectronic circuits (7th ed.). New York: Oxford University Press.

3. ngspice. (n.d.). ngspice user manual. Retrieved October 11, 2026, from https://ngspice.sourceforge.io/docs.html

5  APPENDICES

5.1  Appendix A: SPICE Model and Netlists

The BC238A model was fitted to the datasheet typical values (IS for VBE=0.62 V at 2 mA, βF=170); βR=1, RB=30 Ω and RC=1 Ω approximately reproduce the saturation voltages of the datasheet, and the Early voltage is infinite as required by the experiment sheet. The library model of the BC238 in another simulation program may differ slightly.

Code A.1  BJT model line used in the ngspice simulations

.model BC238A NPN(IS=7.7754e-14 BF=170 BR=1 RB=30 RC=1 CJE=8p CJC=3.5p TF=0.5n)

Netlist A.2  BJT circuit 1 (VBB = 5 V, VCC = 10 V)

Vbb vbb 0 DC 5 Vcc vcc 0 DC 10 RB vbb b 150000 RC vcc c 2200 RE e 0 1000 Q1 c b e BC238A .model BC238A NPN(IS=7.7754e-14 BF=170 BR=1 RB=30 RC=1 CJE=8p CJC=3.5p TF=0.5n) .op .end

Netlist A.3  BJT biasing circuit 2

Vbb vbb 0 DC 5 Vcc vcc 0 DC 12 RB vbb b 22k RC vcc c 1.2k RE e 0 1k Q1 c b e BC238A .model BC238A NPN(IS=7.7754e-14 BF=170 BR=1 RB=30 RC=1 CJE=8p CJC=3.5p TF=0.5n) .op .end