TOBB University of Economics and Technology

Department of Electrical and Electronics Engineering

ELE 224L – Electronic Circuits Laboratory

Experiment 3 – Preliminary Work: MOSFET Amplifiers

| Name Surname |  |
| --- | --- |
| Student ID |  |
| Date |  |

1  INTRODUCTION

1.1  Objective of the Experiment

The objective of this experiment is to learn the operating principle of the MOSFET as an amplifying element in the common-source amplifier and as a matched current source in the current mirror. The DC operating point (Q point), the small-signal gain, and the input and output resistances of a common-source amplifier are calculated for the cases with and without a source bypass capacitor, and the current mirror is analysed for a range of input voltages, including the voltage at which it stops working.

1.2  Benefits of the Experiment

The experiment shows that an amplifier analysis consists of two separate steps (a DC bias analysis and a small-signal AC analysis), that a bypass capacitor changes the gain by a large factor, and that the capacitor values must be checked against the test frequency before a mid-band formula is used. It also shows how a current mirror copies a reference current and where this copying fails. These skills are required for the BJT amplifiers of the next experiments.

2  CALCULATIONS AND SIMULATIONS

2.1  Common-Source Amplifier Circuit

The amplifier circuit is shown in Figure 2.1.1. The values VDD=12 V, R1=47 kΩ, R2=33 kΩ, RD=15 kΩ, RS=10 kΩ, RL=10 kΩ, CC1=22 nF, CC2=2.2 nF and CS=22 nF are used. The BS170 is modelled with the square-law equation ID=KN/2VGS−VTH2 with KN=72 mA/V² and VTH=2.0 V, and the channel-length modulation is neglected.

Figure 2.1.1  Common-source amplifier circuit analysed in the experiment.

DC analysis. All capacitors are open circuits for DC. The gate voltage is given by the divider, (2.1). The gate current is zero, so VGS=VG−IDRS. Substituting this into the square law and writing VOV=VGS−VTH gives the quadratic equation (2.2).

|  | VG=VDDR2R1+R2=12⋅3380=4.95 V | (2.1) |
| --- | --- | --- |

|  | KN2RS VOV2+VOV−VG−VTH=0 ⇒ 360 VOV2+VOV−2.95=0 | (2.2) |
| --- | --- | --- |

The positive root is VOV=0.08914 V, so VGSQ=2.0+0.08914=2.089 V, IDQ=72/20.089142=0.2861 mA, and VDSQ=VDD−IDQRD+RS=12−0.2861×25=4.848 V. Since VDSQ>VOV, the transistor is in saturation, which verifies the assumption. The transconductance is gm=KNVOV=6.418 mS.

Small-signal analysis. For mid-band frequencies, the coupling capacitors are short circuits and the supply is an AC ground. With CS present, the source is at AC ground and the gain is given by (2.3). Without CS, the source resistor produces local feedback, and the gain is given by (2.4). In both cases, Rin=R1∥R2 because the gate draws no current, and Rout=RD because the output resistance of the transistor is infinite in the model (the load RL is not included in Rout).

|  | AV=−gmRD∥RL=−6.418 mS×6 kΩ=−38.51 | (2.3) |
| --- | --- | --- |

|  | AV=−gmRD∥RL1+gmRS=−38.511+6.418×10=−0.591 | (2.4) |
| --- | --- | --- |

|  | Rin=R1∥R2=19.39 kΩ, Rout=RD=15 kΩ | (2.5) |
| --- | --- | --- |

Table 2.1.1  Calculated Q point, gain and resistances of the common-source amplifier

|  | Circuit with CS | Circuit without CS |
| --- | --- | --- |
| VGSQ (V) | 2.089 | 2.089 |
| VDSQ (V) | 4.848 | 4.848 |
| IDQ (mA) | 0.2861 | 0.2861 |
| AV=Vo/Vi (mid-band, CS ideal short) | −38.51 | −0.5908 |
| Rin (kΩ) | 19.39 | 19.39 |
| Rout (kΩ) | 15.00 | 15.00 |
| AV at 10 kHz (phasor analysis) | 7.689 | 0.5671 |

The mid-band formulas assume that CS is a perfect short circuit. This assumption was checked for the test frequency of 10 kHz. At this frequency, the impedance of CS is 1/2πfCS=723 Ω, which is not small compared with 1/gm=156 Ω; the parallel combination of RS and CS is 722 Ω in magnitude, and the gain is reduced by the factor 1+gmZS. A complete phasor analysis of the circuit (including CC1, CC2 and CS) gives AV=7.69 with CS and 0.567 without CS at 10 kHz, as listed in the last row of Table 2.1.1. For AV at 10 kHz to reach 90% of the mid-band value, CS would have to be about 0.28 µF.

SPICE. The circuit was built with the BS170 model. The Q point obtained from the DC analysis is compared with the calculation in Table 2.1.2. A sinusoidal input of 100 mV amplitude and 10 kHz was applied, and the time-domain graphs are shown in Figure 2.1.2 (with CS) and Figure 2.1.3 (without CS); more than two periods are visible. The transient amplitude ratio was measured in steady state. The frequency response obtained with an AC analysis is shown in Figure 2.1.4.

Table 2.1.2  Comparison of the calculation and SPICE for the common-source amplifier (Error = (SPICE − calculation)/calculation)

| Quantity | Calculation | SPICE | Error |
| --- | --- | --- | --- |
| VGSQ (V) | 2.089 | 2.168 | +3.8% |
| VDSQ (V) | 4.848 | 5.044 | +4.0% |
| IDQ (mA) | 0.2861 | 0.2782 | -2.7% |
| AV at 10 kHz, with CS | 7.689 | 7.748 | +0.8% |
| AV at 10 kHz, without CS | 0.5671 | 0.5690 | +0.3% |
| Mid-band AV, with CS (SPICE at 1 MHz) | 38.51 | 48.97 | +27.2% |
| Mid-band AV, without CS (SPICE at 1 MHz) | 0.5908 | 0.5927 | +0.3% |
| Rin at 10 kHz (kΩ) | 19.39 | 19.40 | +0.07% |
| Rout (kΩ) | 15.00 | 15.00 | -0.00% |

Figure 2.1.2  Input and output voltages of the common-source amplifier with CS (100 mV, 10 kHz; ngspice).

Table 2.1.3  Coordinates of the cursors in Figure 2.1.2

| Cursor | x coordinate | y coordinate |
| --- | --- | --- |
| X1 | = 7.525 | vi = 99.98 mV |
| X2 | Time = 7.598 ms | vo = −786.6 mV |

Figure 2.1.3  Input and output voltages of the common-source amplifier without CS (100 mV, 10 kHz; ngspice).

Table 2.1.4  Coordinates of the cursors in Figure 2.1.3

| Cursor | x coordinate | y coordinate |
| --- | --- | --- |
| X1 | = 7.525 | vi = 99.98 mV |
| X2 | Time = 7.520 ms | vo = −56.89 mV |

Figure 2.1.4  Frequency response of the amplifier: SPICE (solid) and the phasor calculation (dashed).

Table 2.1.5  Coordinates of the cursors in Figure 2.1.4

| Cursor | x coordinate | y coordinate |
| --- | --- | --- |
| X1 | Frequency = 1.000e+04 Hz | vo/vi = 17.82 dB |
| X2 | Frequency = 1.000e+04 Hz | vo/vi = −4.898 dB |

Interpretation. The DC values of SPICE differ from the calculation by less than 5%, since the SPICE model has a larger transconductance parameter (KP⋅W/L=123.3 mA/V² against KN=72 mA/V²), a slightly higher threshold (2.1 V against 2.0 V), and series resistances RD and RS of 1.4 and 1.6 Ω. At 10 kHz, the measured gain with CS is 7.75, which is far below the mid-band value 38.5 of the calculation, but it agrees with the phasor calculation (7.69) to 0.8%; the reason is that at this frequency the gain is determined by the impedance of CS, AV≈RL′/ZS, and not by gm. The mid-band gain of SPICE (49.0) is 27% larger than the calculated mid-band gain since the SPICE transistor has a higher gm at the same current, and this difference is visible only above about 100 kHz in Figure 2.1.4. Without CS, the gain is almost independent of gm because gmRS≫1, AV≈−RL′/RS=−0.6, and the SPICE result agrees with the calculation within 0.3%.

2.2  Current Mirror Circuit

The current mirror is shown in Figure 2.2.1. The transistor M1 is diode connected (its gate and drain are tied), so it is always in saturation, and VGS is set by the reference current Iref. Since M2 has the same VGS and is identical to M1, it carries the same current as long as it stays in saturation, i.e., as long as VDS2=VDD−IoutR2≥VOV. The reference current is found from the loop equation (2.6), which is solved for Iref by iteration (or as a quadratic in VOV).

Figure 2.2.1  Current mirror circuit analysed in the experiment.

|  | Vi=R1Iref+VTH+2IrefKN | (2.6) |
| --- | --- | --- |

For example, for Vi=3 V, Vi−VTH=1 V gives the quadratic 24.48 VOV2+VOV−1=0 (with VOV in V and R1=0.68 kΩ), and VOV=0.182 V, Iref=36⋅0.1822=1.20 mA. Then VDS2=15−1.20×1=13.8 V > VOV, so M2 is in saturation and Iout=Iref=1.20 mA. For Vi=1 V (below VTH), M1 is off and both currents are zero. For Vi=15 V, Iref would be 18.08 mA and M2 would require VDS2=15−18.1<0, which is impossible; M2 therefore leaves saturation, enters the triode region, and the solution of the triode current equation together with VDS2=15−IoutR2 gives Iout=14.60 mA. The hand-calculated and simulated values are given in Table 2.2.1.

Table 2.2.1  Reference and output currents of the current mirror: calculation and SPICE

| Vi (V) | Iref calc. (mA) | Iout calc. (mA) | Iref SPICE (mA) | Iout SPICE (mA) | M2 region (calc.) |
| --- | --- | --- | --- | --- | --- |
| 1.00 | 0 | 0 | ≈ 0 | ≈ 0 | cut-off |
| 3.00 | 1.202 | 1.202 | 1.122 | 1.122 | saturation |
| 5.00 | 3.926 | 3.926 | 3.886 | 3.886 | saturation |
| 8.00 | 8.125 | 8.125 | 8.124 | 8.124 | saturation |
| 10.0 | 10.95 | 10.95 | 10.97 | 10.97 | saturation |
| 12.0 | 13.80 | 13.80 | 13.83 | 13.83 | saturation |
| 15.0 | 18.08 | 14.60 | 18.13 | 14.66 | triode |

Figure 2.2.2  Iref–Vi and Iout–Vi graphs of the current mirror (Vi on the x-axis) with the mirror ratio (ngspice).

Table 2.2.2  Coordinates of the cursors in Figure 2.2.2

| Cursor | x coordinate | y coordinate |
| --- | --- | --- |
| X1 | Vi = 10.00 V | Current = 10.97 mA |
| X2 | Vi = 15.00 V | Current = 14.66 mA |

The two currents are equal (within 0.1%) up to Vi=12 V. The SPICE and calculated reference currents agree within 1.0% for Vi≥5 V, whereas at 3 V the SPICE current is 6.6% lower than the calculated one; this larger difference is caused by the different transconductance and threshold of the SPICE model when VOV is small. For Vi=15 V, Iout stays at about 14.7 mA while Iref continues to rise to 18.1 mA, because M2 enters the triode region (the calculation predicts this for Vi>12.4 V; in the 0.5 V steps of the SPICE sweep, the ratio Iout/Iref first falls below 0.99 at 13.0 V). The mirror therefore works as long as the load drop IoutR2 leaves at least VOV across M2, i.e., VDD−IoutR2≥VOV. In saturation, the equality of the currents also relies on the neglect of channel-length modulation, which is zero in the BS170 model.

3  CONCLUSIONS

The calculated Q point of the common-source amplifier (VGSQ=2.09 V, IDQ=0.286 mA, VDSQ=4.85 V) agrees with SPICE within about 4%, with the differences caused by the model parameters. The most important result is that the mid-band gain formulas cannot be applied at 10 kHz with the given capacitor values: the bypass capacitor of 22 nF is too small, and the measured gain with CS is 7.75 instead of 38.5. A bypass capacitor of at least about 0.28 µF would be required for this test frequency, or the test frequency should be raised above about 100 kHz. Without the bypass capacitor, the gain is small (−0.591) but stable, since it depends only on the resistor ratio. The analysis also gave Rin=19.39 kΩ (set by the bias divider) and Rout=15 kΩ (set by RD); the SPICE values agree within 0.1%.

In the current mirror, the output current follows the reference current until the output transistor loses saturation at about 12–13 V; the experiment taught the limit of the mirror (compliance voltage), the use of a diode-connected transistor as a bias generator, and the difference between an ideal copy and the real behaviour. The unexpected results above were explained by checking the capacitor impedances and the model parameters, which is a skill required in the remaining experiments.

4  REFERENCES

1. Sedra, A. S., & Smith, K. C. (2014). Microelectronic circuits (7th ed.). New York: Oxford University Press.

2. ON Semiconductor. (2017). BS170 small signal MOSFET 500 mA, 60 volts [Data sheet]. Retrieved October 11, 2026, from https://www.onsemi.com/download/data-sheet/pdf/bs170-d.pdf

3. ngspice. (n.d.). ngspice user manual. Retrieved October 11, 2026, from https://ngspice.sourceforge.io/docs.html

5  APPENDICES

5.1  Appendix A: SPICE Netlists

Netlist A.1  Common-source amplifier with CS (transient)

VDD vdd 0 DC 12 Vi in 0 DC 0 AC 1 SIN(0 0.1 10k) Cc1 in g 22n R1 vdd g 47k R2 g 0 33k M1 d g s 0 BS170 W=100u L=100u RD vdd d 15k RS s 0 10k CS s 0 22n Cc2 d o 2.2n RL o 0 10k .model BS170 NMOS (VTO=2.1 RS=1.572 RD=1.436 KP=.1233 IS=5p) .tran 2u 8m 0 1u .end

Netlist A.2  Current mirror (DC sweep)

VDD vdd 0 DC 15 Vi vi 0 DC 0 R1 vi d1 680 M1 d1 d1 0 0 BS170 W=100u L=100u M2 d2 d1 0 0 BS170 W=100u L=100u R2 vdd d2 1k .model BS170 NMOS (VTO=2.1 RS=1.572 RD=1.436 KP=.1233 IS=5p) .dc Vi 0 15 0.5 .end